Name: Abdulaziz Abdulrahim

Date: 26/1/26

Project/Part: LM2596 switching regulator

Background Details:

Summary of the purpose of the part/what it is used for:

Use this part instead of the current L7805, the current voltage regulator used does not provide any current limiting. LM2596 provides current limiting, provides a stable 5V if surges occur. LM2596 is also more scalable for future sensors and add ons,

73% efficiency for 3.3V

End deliverables:

Current state of part

Current voltage reg (L7805) will not scale well when sensors are added, too much load will add more heat and increase failure risk and does not support any current limiting.

Your Preliminary concept:

(Method of operation or your part)

LM2596 converts the already protected 12V bus to 5V. Output capacitor smooths the ripple so the load sees a near constant 5V DC supply.

Requirements of your part:

Bulk capacitor near the regulator and near the load.

Potential Validation:

Reduces heat dissipation on the PCB, ensure safe operating temps.

Constraints:

Although much better than current part, the LM2596 is old and inefficient, yet a simple quick fix.

Initial Research:

(Research and comparisons of major parts/methods of operation)

(propose an initial design concept)

Either have 2 bucks where:
12V  Buck 1  5V

12V Buck 2 3v3

OR

12VBuck5V LDO(Low Dropout Regulator) 3v3